# What is Network Topology?

Topology defines the structure of the network of how all the components are interconnected to each other. There are two types of topology: physical and logical topology.

## Types of Network Topology

Physical topology is the geometric representation of all the nodes in a network. There are six types of network topology which are Bus Topology, Ring Topology, Tree Topology, Star Topology, Mesh Topology, and Hybrid Topology.

- The bus topology is designed in such a way that all the stations are connected through a single cable known as a backbone cable.

- Each node is either connected to the backbone cable by drop cable or directly connected to the backbone cable.

- When a node wants to send a message over the network, it puts a message over the network. All the stations available in the network will receive the message whether it has been addressed or not.

- The bus topology is mainly used in 802.3 (ethernet) and 802.4 standard networks.

- The configuration of a bus topology is quite simpler as compared to other topologies.

- The backbone cable is considered as a "single lane" through which the message is broadcast to all the stations.

### Advantages of Bus topology:

- Low-cost cable: In bus topology, nodes are directly connected to the cable without passing through a hub. Therefore, the initial cost of installation is low.

- Moderate data speeds: Coaxial or twisted pair cables are mainly used in bus-based networks that support upto 10 Mbps.

- Familiar technology: Bus topology is a familiar technology as the installation and troubleshooting techniques are well known, and hardware components are easily available.

- Limited failure: A failure in one node will not have any effect on other nodes.

### Disadvantages of Bus topology:

- Extensive cabling: A bus topology is quite simpler, but still it requires a lot of cabling.

- Difficult troubleshooting: It requires specialized test equipment to determine the cable faults. If any fault occurs in the cable, then it would disrupt the communication for all the nodes.

- Signal interference: If two nodes send the messages simultaneously, then the signals of both the nodes collide with each other.

- Reconfiguration difficult: Adding new devices to the network would slow down the network.

- Attenuation: Attenuation is a loss of signal leads to communication issues. Repeaters are used to regenerate the

- Ring topology is like a bus topology, but with connected ends.

- The node that receives the message from the previous computer will retransmit to the next node.

- The data flows in one direction, i.e., it is unidirectional.

- The data flows in a single loop continuously known as an endless loop.

- It has no terminated ends, i.e., each node is connected to other node and having no termination point.

- The data in a ring topology flow in a clockwise direction.

### Advantages of Ring topology:

- Network Management: Faulty devices can be removed from the network without bringing the network down.

- Product availability: Many hardware and software tools for network operation and monitoring are available.

- Cost: Twisted pair cabling is inexpensive and easily available. Therefore, the installation cost is very low.

- Reliable: It is a more reliable network because the communication system is not dependent on the single host computer.

### Disadvantages of Ring topology:

- Difficult troubleshooting: It requires specialized test equipment to determine the cable faults. If any fault occurs in the cable, then it would disrupt the communication for all the nodes.

- Failure: The breakdown in one station leads to the failure of the overall network.

- Reconfiguration difficult: Adding new devices to the network would slow down the network.

- Delay: Communication delay is directly proportional to the number of nodes. Adding new devices increases the communication delay.

- Star topology is an arrangement of the network in which every node is connected to the central hub, switch or a central computer.

- The central computer is known as a server, and the peripheral devices attached to the server are known as clients.

- Coaxial cable or RJ-45 cables are used to connect the computers.

- Hubs or Switches are mainly used as connection devices in a physical star topology.

- Star topology is the most popular topology in network implementation.

### Advantages of Star topology

- Efficient troubleshooting: Troubleshooting is quite efficient in a star topology as compared to bus topology. In a bus topology, the manager has to inspect the kilometers of cable. In a star topology, all the stations are connected to the centralized network. Therefore, the network administrator has to go to the single station to troubleshoot the problem.

- Network control: Complex network control features can be easily implemented in the star topology. Any changes made in the star topology are automatically accommodated.

- Limited failure: As each station is connected to the central hub with its own cable, therefore failure in one cable will not affect the entire network.

- Familiar technology: Star topology is a familiar technology as its tools are cost-effective.

- Easily expandable: It is easily expandable as new stations can be added to the open ports on the hub.

- Cost effective: Star topology networks are cost-effective as it uses inexpensive coaxial cable.

- High data speeds: It supports a bandwidth of approx 100Mbps. Ethernet 100BaseT is one of the most popular Star topology networks.

### Disadvantages of Star topology

- A Central point of failure: If the central hub or switch goes down, then all the connected nodes will not be able to communicate with each other.

- Cable: Sometimes cable routing becomes difficult when a significant amount of routing is required.

- Tree topology combines the characteristics of bus topology and star topology.

- A tree topology is a type of structure in which all the computers are connected with each other in hierarchical fashion.

- The top-most node in tree topology is known as a root node, and all other nodes are the descendants of the root node.

- There is only one path exists between two nodes for the data transmission. Thus, it forms a parent-child hierarchy.

### Advantages of Tree topology

- Support for broadband transmission: Tree topology is mainly used to provide broadband transmission, i.e., signals are sent over long distances without being attenuated.

- Easily expandable: We can add the new device to the existing network. Therefore, we can say that tree topology is easily expandable.

- Easily manageable: In tree topology, the whole network is divided into segments known as star networks which can be easily managed and maintained.

- Error detection: Error detection and error correction are very easy in a tree topology.

- Limited failure: The breakdown in one station does not affect the entire network.

- Point-to-point wiring: It has point-to-point wiring for individual segments.

### Disadvantages of Tree topology

- Difficult troubleshooting: If any fault occurs in the node, then it becomes difficult to troubleshoot the problem.

- High cost: Devices required for broadband transmission are very costly.

- Failure: A tree topology mainly relies on main bus cable and failure in main bus cable will damage the overall network.

- Reconfiguration difficult: If new devices are added, then it becomes difficult to reconfigure.

- Mesh technology is an arrangement of the network in which computers are interconnected with each other through various redundant connections.

- There are multiple paths from one computer to another computer.

- It does not contain the switch, hub or any central computer which acts as a central point of communication.

- The Internet is an example of the mesh topology.

- Mesh topology is mainly used for WAN implementations where communication failures are a critical concern.

- Mesh topology is mainly used for wireless networks.

- Mesh topology can be formed by using the formula:
Number of cables = (n*(n-1))/2;

Where n is the number of nodes that represents the network.

### Advantages of Mesh topology:

Reliable: The mesh topology networks are very reliable as if any link breakdown will not affect the communication between connected computers.

Fast Communication: Communication is very fast between the nodes.

Easier Reconfiguration: Adding new devices would not disrupt the communication between other devices.

### Disadvantages of Mesh topology

- Cost: A mesh topology contains a large number of connected devices such as a router and more transmission media than other topologies.

- Management: Mesh topology networks are very large and very difficult to maintain and manage. If the network is not monitored carefully, then the communication link failure goes undetected.

- Efficiency: In this topology, redundant connections are high that reduces the efficiency of the network.

- The combination of various different topologies is known as Hybrid topology.

- A Hybrid topology is a connection between different links and nodes to transfer the data.

### Advantages of Hybrid Topology

- Reliable: If a fault occurs in any part of the network will not affect the functioning of the rest of the network.

- Scalable: Size of the network can be easily expanded by adding new devices without affecting the functionality of the existing network.

- Flexible: This topology is very flexible as it can be designed according to the requirements of the organization.

- Effective: Hybrid topology is very effective as it can be designed in such a way that the strength of the network is maximized and weakness of the network is minimized.

### Disadvantages of Hybrid topology

- Complex design: The major drawback of the Hybrid topology is the design of the Hybrid network. It is very difficult to design the architecture of the Hybrid network.

- Costly Hub: The Hubs used in the Hybrid topology are very expensive as these hubs are different from usual Hubs used in other topologies.

- Costly infrastructure: The infrastructure cost is very high as a hybrid network requires a lot of cabling, network devices, etc.

# Transmission modes

- The way in which data is transmitted from one device to another device is known as transmission mode.

- The transmission mode is also known as the communication mode.

- Each communication channel has a direction associated with it, and transmission media provide the direction. Therefore, the transmission mode is also known as a directional mode.

- The transmission mode is defined in the physical layer.

The Transmission mode is divided into three categories:

- Simplex mode

- Half-duplex mode

- Full-duplex mode

## Simplex mode

- In Simplex mode, the communication is unidirectional, i.e., the data flow in one direction.

- A device can only send the data but cannot receive it or it can receive the data but cannot send the data.

- This transmission mode is not very popular as mainly communications require the two-way exchange of data. The simplex mode is used in the business field as in sales that do not require any corresponding reply.

- The radio station is a simplex channel as it transmits the signal to the listeners but never allows them to transmit back.

- Keyboard and Monitor are the examples of the simplex mode as a keyboard can only accept the data from the user and monitor can only be used to display the data on the screen.

- The main advantage of the simplex mode is that the full capacity of the communication channel can be utilized during transmission.

### Advantage of Simplex mode:

- In simplex mode, the station can utilize the entire bandwidth of the communication channel, so that more data can be transmitted at a time.

### Disadvantage of Simplex mode:

- Communication is unidirectional, so it has no inter-communication between devices.

## Half-Duplex mode

- In a Half-duplex channel, direction can be reversed, i.e., the station can transmit and receive the data as well.

- Messages flow in both the directions, but not at the same time.

- The entire bandwidth of the communication channel is utilized in one direction at a time.

- In half-duplex mode, it is possible to perform the error detection, and if any error occurs, then the receiver requests the sender to retransmit the data.

- A Walkie-talkie is an example of the Half-duplex mode. In Walkie-talkie, one party speaks, and another party listens. After a pause, the other speaks and first party listens. Speaking simultaneously will create the distorted sound which cannot be understood.

### Advantage of Half-duplex mode:

- In half-duplex mode, both the devices can send and receive the data and also can utilize the entire bandwidth of the communication channel during the transmission of data.

### Disadvantage of Half-Duplex mode:

- In half-duplex mode, when one device is sending the data, then another has to wait, this causes the delay in sending the data at the right time.

## Full-duplex mode

- In Full duplex mode, the communication is bi-directional, i.e., the data flow in both the directions.

- Both the stations can send and receive the message simultaneously.

- Full-duplex mode has two simplex channels. One channel has traffic moving in one direction, and another channel has traffic flowing in the opposite direction.

- The Full-duplex mode is the fastest mode of communication between devices.

- The most common example of the full-duplex mode is a telephone network. When two people are communicating with each other by a telephone line, both can talk and listen at the same time.

### Advantage of Full-duplex mode:

- Both the stations can send and receive the data at the same time.

### Disadvantage of Full-duplex mode:

- If there is no dedicated path exists between the devices, then the capacity of the communication channel is divided into two parts.